Help & FAQ Page for RoboSort: Dynamic Lid-Body Separation System

Welcome to the RoboSort Help Section!

This page provides a detailed guide on how our robotic system works, troubleshooting common issues, and frequently asked questions about 
the lid-body separation process using PyBullet simulation and CNN-based grasp estimation.

🔹 Overview of RoboSort

RoboSort is a digital twin simulation that automates the separation of yogurt cup lids from their respective bodies using the 
Franka-Emika Panda robot. 
Our system integrates CNN-driven grasp point detection, dynamic physics modeling, and inverse kinematics to ensure an efficient and 
scalable solution for recycling facilities.

💡 How It Works

1️.The robot scans the lid’s grasp point using a trained CNN model.
2️.The end-effector (gripper) moves to the grasp location.
3️.The gripper securely holds the lid and detaches it from the cup.
4️.The robot lifts the lid, moves to a designated location, and places it on the floor gently.
5️.The lid’s dynamics are stabilized to prevent it from flying away.
6️.The robot returns to its home position, ready for the next task.

🔹 Step-by-Step Process

Below is a breakdown of how the robot performs lid separation in sequential steps.

🔸 1. Initializing the Simulation

- The PyBullet simulation starts with a ground plane and the Franka-Emika Panda robot.

- The cup and lid models are imported with realistic physics.

- The lid remains constrained to the cup until the grasp operation is successful.

🔸 2. Gripper Setup & Movement

✅ Gripper Opens → The robot first opens its gripper to prepare for grasping.
✅ Moves to Grasp Position → It moves slightly above the grasp position for a safe approach.
✅ Moves Down to Lid → It lowers carefully to reach the lid’s grasping area.

🔸 3. Lid Grasping & Holding

✅ Closes Gripper → The gripper closes gradually with increasing force.
✅ Applies Constant Holding Force → This prevents slipping during movement.
✅ Verifies Grasp → The robot checks if it successfully grasped the lid using contact points.

🔸 4. Lid Lifting & Placement

✅ Lifts the Lid → The robot lifts the lid upwards to avoid collision with the cup.
✅ Moves to Drop Position → The robot rotates and moves to the desired drop location.
✅ Lowers Lid Gently → It gradually lowers the lid to prevent bouncing.
✅ Releases Lid → The gripper opens slowly to ensure a stable release.

🔸 5. Stabilization & Reset

✅ Prevents Lid Flying Away → The system increases lid mass & friction to avoid unintended motion.
✅ Resets Robot to Home Position → The robot moves back and resets for the next iteration.

🔹 Frequently Asked Questions (FAQ)

💬 1. Why is my lid flying away after placement?

✔️ The lid isn’t stabilizing properly due to low mass or friction settings.
✔️ Fix: Increase the lid’s mass and apply p.resetBaseVelocity(lid, [0, 0, 0], [0, 0, 0]).

💬 2. Why is the gripper passing through the lid?

✔️ The lid collision properties may not be defined correctly.
✔️ Fix: Ensure the GEOM_FORCE_CONCAVE_TRIMESH flag is set when loading the lid.

💬 3. How can I make the robot grip the lid more tightly?

✔️ Increase the closing force of the gripper using:

p.setJointMotorControl2(robot, left_finger, p.POSITION_CONTROL, targetPosition=0.005, force=7000)

p.setJointMotorControl2(robot, right_finger, p.POSITION_CONTROL, targetPosition=0.005, force=7000)

✔️ Also, apply a holding force to maintain grip.

💬 4. Why is the robot colliding with the cup when lifting?

✔️ The lifting trajectory may not be high enough.
✔️ Fix: Modify the lift step by adding extra vertical clearance before moving:

lift_height = 0.20  # Increase from 0.10 to avoid collision

💬 5. Can I rotate the lid on its own axis?

✔️ Yes! You can rotate the lid around its Z-axis using:

p.resetBasePositionAndOrientation(lid, [x, y, z], p.getQuaternionFromEuler([0, 0, rotation_angle]))

✔️ Modify rotation_angle dynamically for continuous rotation.

💬 6. How do I keep the simulation running continuously?

✔️ Add an infinite loop to maintain real-time visualization:

while True:

p.stepSimulation()

time.sleep(1/240.0)  # Adjust step time

💬 7. How can I change the lid model dynamically?

✔️ Pass different lid paths to setup_environment().
✔️ Use a GUI button to switch between Lid_15.obj, Lid_60.obj, Lid_120.obj, etc.

💬 8. Why is the lid position changing unexpectedly?

✔️ The lid’s physics properties might be unstable.
✔️ Fix: Set a fixed initial position and ensure it's not drifting.

🔹 Software & Model Information

📌 Simulation Tool → PyBullet
📌 Robot → Franka-Emika Panda
📌 Machine Learning Model → CNN-based grasp point estimation
📌 Physics Engine → PyBullet with realistic collision handling
📌 Programming Language → Python

🔹 Future Improvements

🚀 Better grasp point detection using real-world datasets.
🚀 Integration with industrial recycling facilities for deployment.
🚀 Enhanced real-time visualization for user interaction.

🔹 Need More Help?

📧 Contact Us at support@robosort.com for more assistance.